AHD-XM330A1-0062-32X32-S-V1.02 Interface Description

J24    1

2

3

3

J231

2

3

4

5

6

CN2  1

2    3

CN4

2

1

Detailed Definition

| Socket | Specific No. | Interface Description | Function |
| --- | --- | --- | --- |
| J24 | 1 | IRC | Infrared led Input |
|  | 2 | GND | Ground |
|  | 3 | NC | Undefined |
| J23 | 1 | GND | Ground |
|  | 2 | NC | Undefined |
|  | 3 | PAL/NTSC | Video Format Switch |
|  | 4 | NC | Undefined |
|  | 5 | CVI | Switch to CVI by grounding |
|  | 6 | TVI | Switch to TVI by grounding |
| CN2 | 1 | VIDEO | Video output |
|  | 2 | GND | Ground |
|  | 3 | 12V | 12V DC input |
| CN4 | 1 | ICR control signal | output |
|  | 2 | ICR control signal | output |