AHD-XM330A1-323-32X32-S-V1.01 Interface Description

J266  5

4

3

2

1

J24    1

2

3

3

CN2  1

2    3

J236

5

4

3

2

1

CN4  1  2

2   1

Detailed Definition

| Socket | Specific No. | Interface Description | Function |
| --- | --- | --- | --- |
| J24 | 1 | IRC | Infrared led Input |
|  | 2 | GND | Ground |
|  | 3 | NC | Undefined |
| J23 | 1 | GND | Ground |
|  | 2 | NC | Undefined |
|  | 3 | PAL/NTSC | Video Standard Switch |
|  | 4 | NC | Undefined |
|  | 5 | CVI | Switch to CVI by grounding |
|  | 6 | TVI | Switch to TVI by grounding |
| CN2 | 1 | VIDEO | Video output |
|  | 2 | GND | Ground |
|  | 3 | 12V | 12V DC input |
| CN4 | 1 | ICR control signal | output |
|  | 2 | ICR control signal | output |
| J26 | 1 | NC | Undefined |
|  | 2 | GND | Ground |
|  | 3 | TXD | Serial port transmit |
|  | 4 | RXD | Serial port receive |
|  | 5 | SWITCH | Program debug switch |
|  | 6 | RSTN | reset |