# AHG模块-AHD-XM330V200-0D25-32X32-S V2.01 Interface Description

CN5  1 2

CN2   1 2 3

J2

1

2

3

4

J23   4 3 2 1

J24  3 2 1

Detailed Definition：

| Socket |  | Specific No. | Interface Description | Function |  |
| --- | --- | --- | --- | --- | --- |
| CN2 |  | 1 | CVBS | Video Out |  |
|  |  | 2 | GND | Ground |  |
|  |  | 3 | 12V | 12V DC input |  |
| CN5 |  | 1 | IRCUTA | IRCUT |  |
|  |  | 2 | IRCUTB |  |  |
| J24 |  | 1 | ADC1VIN | Photosensitive signal |  |
|  |  | 2 | GND | Ground |  |
|  |  | 3 | NC | Undefined |  |
| J2 |  | 1 | R_N_P | Red CTRL |  |
|  |  | 2 | GND | Ground |  |
|  |  | 3 | 3V3 | 3.3V DC input |  |
| J23 | 1 |  | GND |  | Ground |
|  | 2 |  | CVI |  | Switch to CVI by grounding |
|  | 3 |  | WL_N_P |  | White light CTRL |
|  | 4 |  | XVI |  | Switch to XVI by grounding |