AHG-CMOS0222-38X38-V1_01 Interface Description

P3 1 2 3

P4

1  2

P16  5

4

3

2

1

P2 3 2 1

Detailed Definition

| Label | Socket | Specific No. | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J4 | 1 | GND | Refer to the attachment |
|  |  | 2 | LEFT |  |
|  |  | 3 | RIGHT |  |
|  |  | 4 | SET |  |
|  |  | 5 | UP |  |
|  |  | 6 | DOWN |  |
| P2 | J3 | 1 | 红外灯控制信号输入 | Signal Input |
|  |  | 2 | GND | Ground |
|  |  | 3 | NC | Undefined |
| P3 | J2 | 1 | VIDEO | Video output |
|  |  | 2 | GND | Ground |
|  |  | 3 | +12V | 12V DC input |
| P4 | J1 | 1 | ICR control signal | output |
|  |  | 2 | ICR control signal | output |

Attachment：P1(J1) function description:

remarks: functions can be supported only with the latest firmware(version 20140814& after that)

- With OSD key tail wire：

| Label | Socket | Specific No. | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J1 | 1 | GND | Ground |
|  |  | 2 | LEFT | OSD Menu left key |
|  |  | 3 | RIGHT | OSD Menu right key |
|  |  | 4 | SET | OSD Menu OK key |
|  |  | 5 | UP | OSD Menu up key |
|  |  | 6 | DOWN | OSD Menu down key |

Long-press shortcut：

- long press LEFT key：CVBS、AHD_M Switch（alternate）

- long press RIGHT key：AHD_H

- long press DOWN key：Switch NTSC、PAL

remarks：long press to switch, once switch successfully, please release the key(or restart after successful switch)

- With PAL short circuit tail wire(new)

- There is already a short circuit between Pins No. 1&3

- Short circuit between 1&3 only(default) AHD_H mode

c.  Short circuit between 2 and 1&3,CVBS mode

d.  Short circuit between 4 and 1&3，AHD_M mode

| Label | Socket | Specific No. | Function |
| --- | --- | --- | --- |
| P1 | J1 | 1 | Ground |
|  |  | 2 | CVBS |
|  |  | 3 | Connected to No.1 pin（GND） |
|  |  | 4 | AHD_M |
|  |  | 5 |  |
|  |  | 6 |  |

- With NTSC short circuit tail wire

- There is already a short circuit between Pins No. 1&2

- Short circuit between 1&2 only(default) AHD_H mode

- Short circuit between 5 and 1&2,CVBS mode

- Short circuit between 6 and 1&2，AHD_M mode

| Label | Socket | Specific No. | Function |
| --- | --- | --- | --- |
| P1 | J1 | 1 | Ground |
|  |  | 2 | Connected to No.1 pin（GND） |
|  |  | 3 |  |
|  |  | 4 |  |
|  |  | 5 | CVBS |
|  |  | 6 | AHD_M |