BLK510-0062-WIFI-SD-38X38-V1_01 Interface Description

J29

24-1（L→R）

J8

1

2

3

J10

This interface is not welded

J5

1

2

3

4

3

J11 This interface is not welded

J12 This interface is not welded

J7

2 1

Detailed Definition

| Label | Socket | Specific No | Interface Description |
| --- | --- | --- | --- |
| J5 | 1 | NC | Undefined |
|  | 2 | NC | Undefined |
|  | 3 | GND | GND |
|  | 4 | 5V | 5V DC input |
| J8 | 1 | LED_CTRL | Infrared input |
|  | 2 | GND | GND |
|  | 3 | PWR_VIN | DC input |
| J7 | 1 | IRCUT_A | IRCUT |
|  | 2 | IRCUT_B |  |
| J12 | 1 | SPKN | SPK |
|  | 2 | SPKP |  |
| J29 | 1 | 3V3 | 3.3V |
|  | 2 | 3V3 | 3.3V |
|  | 3 | GND | GND |
|  | 4 | VBAT |  |
|  | 5 | ALARM_IN | ALARM_IN |
|  | 6 | GPIO_07 | Key-set |
|  | 7 | GPIO46 | LED |
|  | 8 | ALARM_OUT | ALARM_OUT |
|  | 9 | GND | GND |
|  | 10 | WIFI_DATA0 | WIFI_DATA0 |
|  | 11 | WIFI_PDN | WIFI_PDN |
|  | 12 | GND | GND |
|  | 13 | WIFI_CMD | WIFI_CMD |
|  | 14 | GND | GND |
|  | 15 | WIFI_CLK | WIFI_CLK |
|  | 16 | GND | GND |
|  | 17 | SD_DATA3 | SD_DATA3 |
|  | 18 | SD_DATA2 | SD_DATA2 |
|  | 19 | SD_DATA1 | SD_DATA1 |
|  | 20 | SD_DATA0 | SD_DATA0 |
|  | 21 | GND | GND |
|  | 22 | SD_CMD | SD_CMD |
|  | 23 | GND | GND |
|  | 24 | SD_CLK | SD_CLK |
| J11 | 1 | MIC1 | MIC |
|  | 2 | MIC2 |  |
| J10 | 1 | NC | Undefined |
|  | 2 | GND | GND |
|  | 3 | UART0_TXD | UART0_TXD |
|  | 4 | UART0_RXD | UART0_RXD |
|  | 5 | SWTICH_KEY | SWTICH_KEY |
|  | 6 | EXT_RSTN | EXT_RSTN |

BLK510-38X38-PORT-WP-V1_01 Interface Description

J10

This interface is not welded

J8

This interface is not welded

J9

This interface is not welded

J5

This interface is not welded

3

J2

1-24（L→R）

详细定义

| sign | Socket | Specific No | Interface Description |
| --- | --- | --- | --- |
| J5 | 1 | GPIO46 | LED |
|  | 2 | GND | GND |
|  | 3 | GND | GND |
|  | 4 | ALARM_OUT1 | ALARM_OUT |
| J8（预留） | 1 | VBAT | VBAT |
|  | 2 | GND | GND |
| J9 | 1 | ALARM_IN | ALARM_IN |
|  | 2 | GND | GND |
| J10（预留） | 1 | GPIO_07 |  |
|  | 2 | GND | GND |
| J2 | 1 | 3V3 | 3V3 |
|  | 2 | 3V3 | 3V3 |
|  | 3 | GND | GND |
|  | 4 | VBAT | VBAT |
|  | 5 | ALARM_IN | ALARM_IN |
|  | 6 | GPIO_07 | GPIO_07 |
|  | 7 | GPIO46 | GPIO46 |
|  | 8 | ALARM_OUT | ALARM_OUT |
|  | 9 | GND | GND |
|  | 10 | WIFI_DATA0 | WIFI_DATA0 |
|  | 11 | WIFI_PDN | WIFI_PDN |
|  | 12 | GND | GND |
|  | 13 | WIFI_CMD | WIFI_CMD |
|  | 14 | GND | GND |
|  | 15 | WIFI_CLK | WIFI_CLK |
|  | 16 | GND | GND |
|  | 17 | SD_DATA3 | SD_DATA3 |
|  | 18 | SD_DATA2 | SD_DATA2 |
|  | 19 | SD_DATA1 | SD_DATA1 |
|  | 20 | SD_DATA0 | SD_DATA0 |
|  | 21 | GND | GND |
|  | 22 | SD_CMD | SD_CMD |
|  | 23 | GND | GND |
|  | 24 | SD_CLK | SD_CLK |

J1 This interface is not welded

| sign | Socket | Specific No | Interface Description |
| --- | --- | --- | --- |
| J1 | / | SD Card | SD Card |