BLK510FX1-0062- 38X38-WP-V1_01 Interface Description

J1

1 2 3 4 5 6

J7

1 2 3 4

J4

1

2

3

4

5

6

J8

1-24

J3

This interface is not welded

J2

This interface is not welded

J6

3

2

1

Detailed Definition

| Label | Socket | Specific No | Interface Description |
| --- | --- | --- | --- |
| J1 | 1 | ETHRX- | ETHRX- |
|  | 2 | ETHRX+ | ETHRX+ |
|  | 3 | PHY_AD0 | PHY_AD0 |
|  | 4 | ETHTX- | ETHTX- |
|  | 5 | ETHTX+ | ETHTX+ |
|  | 6 | PHY_AD3 | PHY_AD3 |
| J7 | 1 | NC | Undefined |
|  | 2 | NC | Undefined |
|  | 3 | GNG | GND |
|  | 4 | 12V | 12V DC input |
| J6 | 1 | LED_CTRL | Infrared input |
|  | 2 | GND | GND |
|  | 3 | NC | DC input |
| J4 | 1 | NC | NC |
|  | 2 | GND | GND |
|  | 3 | UART0_TXD | UART0_TXD |
|  | 4 | UART0_RXD | UART0_RXD |
|  | 5 | SWTICH_PROG | SWTICH_PROG |
|  | 6 | EXT_RXTN | EXT_RXTN |
| J3 | 1 | MICIN | MIC |
|  | 2 | MICIN |  |
| J2 | 1 | SPKP | SPK |
|  | 2 | SPKN |  |
| J3 | 1 | 3V3 | 3.3V |
|  | 2 | 3V3 | 3.3V |
|  | 3 | GND | GND |
|  | 4 | VBAT |  |
|  | 5 | ALARM_IN | ALARM_IN |
|  | 6 | GPIO_07 | Key-set |
|  | 7 | GPIO46 | LED |
|  | 8 | ALARM_OUT | ALARM_OUT |
|  | 9 | GND | GND |
|  | 10 | WIFI_DATA0 | WIFI_DATA0 |
|  | 11 | WIFI_PDN | WIFI_PDN |
|  | 12 | GND | GND |
|  | 13 | WIFI_CMD | WIFI_CMD |
|  | 14 | GND | GND |
|  | 15 | WIFI_CLK | WIFI_CLK |
|  | 16 | GND | GND |
|  | 17 | SD_DATA3 | SD_DATA3 |
|  | 18 | SD_DATA2 | SD_DATA2 |
|  | 19 | SD_DATA1 | SD_DATA1 |
|  | 20 | SD_DATA0 | SD_DATA0 |
|  | 21 | GND | GND |
|  | 22 | SD_CMD | SD_CMD |
|  | 23 | GND | GND |
|  | 24 | SD_CLK | SD_CLK |

J5

1

2

| J3 | 1 | IRCUT1 | IRCUT |
| --- | --- | --- | --- |
|  | 2 | IRCUT2 |  |

BLK510-38X38-PORT-WP-V2_01

BLK510-38X19-PORT-WP-V2_01 Interface Description

J10

1 2

J2

24-1

U1

J9（预留）

2

1

| J2 | 1 | 3V3 | 3.3V |
| --- | --- | --- | --- |
|  | 2 | 3V3 | 3.3V |
|  | 3 | GND | GND |
|  | 4 | VBAT | VBAT |
|  | 5 | ALARM_IN | ALARM_IN |
|  | 6 | GPIO_07 | GPIO_07 |
|  | 7 | GPIO46 | GPIO46 |
|  | 8 | ALARM_OUT | ALARM_OUT |
|  | 9 | GND | GND |
|  | 10 | WIFI_DATA0 | WIFI_DATA0 |
|  | 11 | WIFI_PDN | WIFI_PDN |
|  | 12 | GND | GND |
|  | 13 | WIFI_CMD | WIFI_CMD |
|  | 14 | GND | GND |
|  | 15 | WIFI_CLK | WIFI_CLK |
|  | 16 | GND | GND |
|  | 17 | SD_DATA3 | SD_DATA3 |
|  | 18 | SD_DATA2 | SD_DATA2 |
|  | 19 | SD_DATA1 | SD_DATA1 |
|  | 20 | SD_DATA0 | SD_DATA0 |
|  | 21 | GND | GND |
|  | 22 | SD_CMD | SD_CMD |
|  | 23 | GND | GND |
|  | 24 | SD_CLK | SD_CLK |
| U1 | / | / | / |
| J9 | 1 | AL_IN1 | AL_IN1 |
|  | 2 | GND | GND |
| J10 | 1 | GPIO_07 | GPIO_07 |
|  | 2 | GND | GND |