BRD_XMDZ-CZ-LED-KEY-FPC-RING-V1_01

P13

P1

1 2 3 4 5 6

| sign | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P1 | J1 | 1 | 3.3V | 3.3V Power input |
|  |  | 2 | GND | GND |
|  |  | 3 | GND | GND |
|  |  | 4 | SET_GPIO03 | Doorbell button signal |
|  |  | 5 | MCU_LED2 | Led control signal 1 |
|  |  | 6 | MCU_LED1 | Led control signal 2 |
| P13 | K1 | 1 | GND | GND |
|  |  | 2 | GND | GND |
|  |  | 3 | SET_GPIO03 | Doorbell button signal |
|  |  | 4 | SET_GPIO03 | Doorbell button signal |

BRD_XMDZ-CZ-POWER-SD-USB-433-RING-V2_01

P3

1 2 3

P14

P4

P15

P2

1 2 3

P5

23 21……..5 3 1

24 22……..6 4 2

| sign | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P2 | J1 | 1 | AC24+ | AC24 V+ input |
|  |  | 2 | AC24+ | AC24V+ input |
|  |  | 3 | AC24+ | AC24V+ input |
| P3 | J2 | 1 | AC24- | AC24V- input |
|  |  | 2 | AC24- | AC24V- input |
|  |  | 3 | AC24- | AC24V- input |
| P4 | J3 | 1 | +5V | USB5V Power input |
|  |  | 2 | Nc | / |
|  |  | 3 | Nc | / |
|  |  | 4 | Nc | / |
|  |  | 5 | Gnd | Gnd |
|  |  | 6 | Gnd | Gnd |
|  |  | 7 | Gnd | Gnd |
| P5 | J7 | 1 | +5V | 5V Power input |
|  |  | 2 | +5V | 5V Power input |
|  |  | 3 | Gnd | Gnd |
|  |  | 4 | +5V | 5V Power input |
|  |  | 5 | Gnd | Gnd |
|  |  | 6 | Gnd | Gnd |
|  |  | 7 | SD_ EN | SD_ EN |
|  |  | 8 | CARD_DETECT | CARD_DETECT |
|  |  | 9 | SDIO1_CDATA1 | SDIO1_CDATA1 |
|  |  | 10 | SDIO1_CDATA0 | SDIO1_CDATA0 |
|  |  | 11 | SDIO1_CDATA3 | SDIO1_CDATA3 |
|  |  | 12 | SDIO1_CDATA2 | SDIO1_CDATA2 |
|  |  | 13 | Gnd | Gnd |
|  |  | 14 | SDIO1_CCMD | SDIO1_CCMD |
|  |  | 15 | 433_IN | 433_IN |
|  |  | 16 | Gnd | Gnd |
|  |  | 17 | SW_ON/OFF | SW_ON/OFF |
|  |  | 18 | SDIO1_CCLK_OUT | SDIO1_CCLK_OUT |
|  |  | 19 | SET_GPIO04 | SET_GPIO04 |
|  |  | 20 | Gnd | Gnd |
|  |  | 21 | Gnd | Gnd |
|  |  | 22 | Gnd | Gnd |
|  |  | 23 | +3.3V | +3.3V |
|  |  | 24 | SDIO1_VOUT | SDIO1_VOUT |
| P14 | K1 | 1 | Gnd | Gnd |
|  |  | 2 | Gnd | Gnd |
|  |  | 3 | SW_ON/OFF | Restore factory set key signal |
|  |  | 4 | SW_ON/OFF | Restore factory set key signal |
| P15 | K2 | 1 | Gnd | Gnd |
|  |  | 2 | SET_GPIO04 | Anti disassembly key signal |

BRD_XMDZ-CZ-BLK18EV3-0307E-RING-V1_01

P2

1 2 3

P6

2 1

P8

1 2 3

P7

1

2

P9

2 1

P10

2 1

P11

23 21……..5 3 1

24 22……..6 4 2

P12

1 2 3 4 5 6

| sign | Socket | Specific No | Interface Description | Function |
| --- | --- | --- | --- | --- |
| P6 | J1 | 1 | VOP | Horn signal+ |
|  |  | 2 | VON | Horn signal- |
| P7 | J2 | 1 | IR_CUT1 | IR_CUT signal 1 |
|  |  | 2 | IR_CUT2 | IR_CUT signal 2 |
| P8 | J4 | 1 | UART0_RXD | UART0_RXD |
|  |  | 2 | UART0_TXD | UART0_TXD |
|  |  | 3 | Gnd | Gnd |
| P9 | J7 | 1 | +5V_BQ | +5V_BQ |
|  |  | 2 | Gnd | Gnd |
| P10 | J9 | 1 | GND | GND |
|  |  | 2 | MIC_P | Microphone signal |
| P11 | J14 | 1 | +3.3V | 3.3V Power output |
|  |  | 2 | +3.3V | 3.3V Power output |
|  |  | 3 | Gnd | Gnd |
|  |  | 4 | Gnd | Gnd |
|  |  | 5 | SET_GPIO04 | Anti disassembly key signal |
|  |  | 6 | Gnd | Gnd |
|  |  | 7 | SW_ON/OFF1 | Restore factory set key signal |
|  |  | 8 | SD0_CLK/SFC_INPUT_SEL | SD clock |
|  |  | 9 | 433_IN | 433 signal |
|  |  | 10 | Gnd | Gnd |
|  |  | 11 | Gnd | Gnd |
|  |  | 12 | SD0_CMD | SD0_CMD |
|  |  | 13 | SD0_D3 | SD0_D3 |
|  |  | 14 | SD0_D2 | SD0_D2 |
|  |  | 15 | SD0_D1 | SD0_D1 |
|  |  | 16 | SD0_D0 | SD0_D0 |
|  |  | 17 | SD_EN | SD_EN |
|  |  | 18 | SD0_CARD_DETECT | SD0_CARD_DETECT |
|  |  | 19 | Gnd | Gnd |
|  |  | 20 | Gnd | Gnd |
|  |  | 21 | Gnd | Gnd |
|  |  | 22 | +5V_IN | +5V_IN |
|  |  | 23 | +5V_IN | +5V_IN |
|  |  | 24 | +5V_IN | +5V_IN |
| P12 | J16 | 1 | +3.3V_PIR | +3.3V_PIR |
|  |  | 2 | GND | GND |
|  |  | 3 | GND | GND |
|  |  | 4 | SET_GPIO03 | SET_GPIO03 |
|  |  | 5 | MCU_RLED | MCU_RLED |
|  |  | 6 | MCU_BED1 | MCU_BED1 |