CN5  1 2

AHD-XM330V200-0235P-32X32 V3.01 Interface Description

CN2   1 2 3

J23   4 3 2 1

J24  3 2 1

J2

3

2

1

- Detailed Definition：

| Socket |  | Specific No. | Interface Description | Function |  |
| --- | --- | --- | --- | --- | --- |
| CN2 |  | 1 | CVBS | Video Out |  |
|  |  | 2 | GND | Ground |  |
|  |  | 3 | 12V | 12V DC input |  |
| CN5 |  | 1 | IRCUTA | IRCUT |  |
|  |  | 2 | IRCUTB |  |  |
| J24 |  | 1 | ADC1VIN | Red_Ctrl |  |
|  |  | 2 | GND | Ground |  |
|  |  | 3 | NC | Undefined |  |
| J2 |  | 1 | ALARM_IN1 | White light CTRL |  |
|  |  | 2 | GND | Ground |  |
|  |  | 3 | 3.3V | 3.3V DC input |  |
| J23 | 1 |  | GND |  | Ground |
|  | 2 |  | CVI |  | Switch to CVI by grounding |
|  | 3 |  | MODE_N-P |  | Red_CTRL |
|  | 4 |  | TVI |  | Switch to TVI by grounding |